《数字逻辑》实验报告

| 姓名 |  | 孙莹莹 |  | 年级 |  | 2020级 |
| --- | --- | --- | --- | --- | --- | --- |
| 学号 |  | 20204104 |  | 专业、班级 |  | 计算机科学与技术卓越一班 |
| 实验名称 | 实验八 乘除法器的实现 |  |  |  |  |  |
| 实验时间 | 2021.11.15 |  | 实验地点 |  | DS1410 |  |
| 实验成绩 |  |  | 实验性质 |  | □验证性 □设计性 □综合性 |  |
| 教师评价： □算法/实验过程正确； □源程序/实验内容提交 □程序结构/实验步骤合理； □实验结果正确； □语法、语义正确； □报告规范； 评语： 评价教师签名（电子签名）： |  |  |  |  |  |  |
| 一、实验目的 设计实现一个8位的乘法器和除法器，并进行仿真验证和下板验证。 |  |  |  |  |  |  |
| 二、实验项目内容 1.乘法器 （1）乘法器设计 （2）乘法器代码实现 （3）乘法器仿真 （4）乘法器下板验证 2.除法器 （1）除法器设计 （2）除法器代码实现 （3）除法器仿真 （4）除法器下板验证 |  |  |  |  |  |  |
| - 实验设计（实验原理、真值表、原理图等） - 1.乘法器 - （1）实验原理 两个8位整数相乘，乘积不会超过16位。假设乘数为x，被乘数为y，乘积为z，两个8位整数相乘，实际上进行了8次加法操作，乘法实现过程如下所示。每次判断y的0位是否为1，若为1则z加上x；反之若y的0位不为1，则不执行操作。每次判断后x左移一位，y右移一位。重复以上步骤8次，完成乘法操作。 （2）乘法器原理图 2.除法器 （1）实验原理 假设被除数为a，除数为b，均是8位整数，则它们相除的商和余数一不会超过8位。除法器其实是通过减法和移位操作实现除法功能的，实现过程如下。 首先将a转换为16位数tempa，tempa高8位为0，低8位为a；将b转换为tempb，tempb高8位为b，低8位为0。每个周期开始之前将tempa左移一位，然后跟tempb比较，若大于tempb，则tempa赋值为tempa-tempb+1，否则继续往下执行。重复以上比较执行的步骤8次，完成除法操作。最后tempa高8位是余数，低8位是商。 （2）除法器原理图 |  |  |  |  |  |  |
| 四、实验过程或算法 1.乘法器 （1）乘法器核心代码 module mutl_machine #(parameter SIZE = 8) (a,b,out); input wire [SIZE-1:0] a; input wire[SIZE-1:0] b; output reg [2*SIZE-1:0] out; reg [2*SIZE-1:0] tempa; reg [SIZE-1:0] tempb; always@(*) begin tempa = a; tempb = b; out = 16'b0000_0000_0000_0000; repeat(SIZE) begin if(tempb[0]) out = out + tempa; tempa = {tempa[14:0],1'b0}; tempb = {1'b0, tempb[7:1]}; end end endmodule （2）七段译码管控制代码 module show( input clk, input [15:0] data, output [3:0] sm_wei, output [7:0] sm_duan ); //分频 integer clk_cnt; reg clk_400Hz; always@(posedge clk) begin if(clk_cnt==32'd100000) begin clk_cnt<=1'b0; clk_400Hz<=~clk_400Hz; end else begin clk_cnt<=clk_cnt+1'b1; end end //位控制 reg [3:0]wei_ctrl=4'b1110; //共阳极 always@(posedge clk_400Hz) begin wei_ctrl<={wei_ctrl[2:0],wei_ctrl[3]}; end //段控制 reg[3:0] duan_ctrl; always@(wei_ctrl)begin case(wei_ctrl) 4'b1110:duan_ctrl=data[3:0]; 4'b1101:duan_ctrl=data[7:4]; 4'b1011:duan_ctrl=data[11:8]; 4'b0111:duan_ctrl=data[15:12]; default:duan_ctrl=4'hf; endcase end //解码模块 reg[7:0] duan; always@(duan_ctrl) begin case(duan_ctrl) 4'h0:duan=8'b1100_0000; //h-a 4'h1:duan=8'b1111_1001; 4'h2:duan=8'b1010_0100; 4'h3:duan=8'b1011_0000; 4'h4:duan=8'b1001_1001; 4'h5:duan=8'b1001_0010; 4'h6:duan=8'b1000_0010; 4'h7:duan=8'b1111_1000; 4'h8:duan=8'b1000_0000; 4'h9:duan=8'b1001_0000; 4'ha:duan=8'b1000_1000; 4'hb:duan=8'b1000_0011; 4'hc:duan=8'b1100_0110; 4'hd:duan=8'b1010_0001; 4'he:duan=8'b1000_0110; 4'hf:duan=8'b1000_1110; default:duan=8'b1100_0000; endcase end assign sm_wei = wei_ctrl; assign sm_duan = duan; endmodule （3）顶层模块调用乘法器和七段译码管 module multitude( input clk, input wire [7:0] a, input wire [7:0] b, output [15:0] out, output [3:0] sm_wei, output [7:0] sm_duan ); mutl_machine tryMachine(a,b,out); show tryShow(clk,out,sm_wei,sm_duan); endmodule （4）管脚约束文件 set_property IOSTANDARD LVCMOS33 [get_ports {a[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {a[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {a[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {a[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {a[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {a[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {a[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {a[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[15]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[14]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[13]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[12]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[11]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[10]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[9]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[8]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {out[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {sm_duan[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {sm_duan[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {sm_duan[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {sm_duan[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {sm_duan[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {sm_duan[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {sm_duan[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {sm_duan[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {sm_wei[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {sm_wei[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {sm_wei[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {sm_wei[0]}] set_property PACKAGE_PIN W7 [get_ports {sm_duan[0]}] set_property PACKAGE_PIN W6 [get_ports {sm_duan[1]}] set_property PACKAGE_PIN U8 [get_ports {sm_duan[2]}] set_property PACKAGE_PIN V8 [get_ports {sm_duan[3]}] set_property PACKAGE_PIN U5 [get_ports {sm_duan[4]}] set_property PACKAGE_PIN V5 [get_ports {sm_duan[5]}] set_property PACKAGE_PIN U7 [get_ports {sm_duan[6]}] set_property PACKAGE_PIN V7 [get_ports {sm_duan[7]}] set_property PACKAGE_PIN U2 [get_ports {sm_wei[0]}] set_property PACKAGE_PIN U4 [get_ports {sm_wei[1]}] set_property PACKAGE_PIN V4 [get_ports {sm_wei[2]}] set_property PACKAGE_PIN W4 [get_ports {sm_wei[3]}] set_property PACKAGE_PIN R2 [get_ports {a[0]}] set_property PACKAGE_PIN T1 [get_ports {a[1]}] set_property PACKAGE_PIN U1 [get_ports {a[2]}] set_property PACKAGE_PIN W2 [get_ports {a[3]}] set_property PACKAGE_PIN R3 [get_ports {a[4]}] set_property PACKAGE_PIN V2 [get_ports {a[7]}] set_property PACKAGE_PIN T3 [get_ports {a[6]}] set_property PACKAGE_PIN T2 [get_ports {a[5]}] set_property PACKAGE_PIN W13 [get_ports {b[0]}] set_property PACKAGE_PIN W14 [get_ports {b[1]}] set_property PACKAGE_PIN V15 [get_ports {b[2]}] set_property PACKAGE_PIN W15 [get_ports {b[3]}] set_property PACKAGE_PIN W17 [get_ports {b[4]}] set_property PACKAGE_PIN W16 [get_ports {b[5]}] set_property PACKAGE_PIN V16 [get_ports {b[6]}] set_property PACKAGE_PIN V17 [get_ports {b[7]}] set_property PACKAGE_PIN L1 [get_ports {out[0]}] set_property PACKAGE_PIN P1 [get_ports {out[1]}] set_property PACKAGE_PIN N3 [get_ports {out[2]}] set_property PACKAGE_PIN P3 [get_ports {out[3]}] set_property PACKAGE_PIN U3 [get_ports {out[4]}] set_property PACKAGE_PIN W3 [get_ports {out[5]}] set_property PACKAGE_PIN V3 [get_ports {out[6]}] set_property PACKAGE_PIN V13 [get_ports {out[7]}] set_property PACKAGE_PIN V14 [get_ports {out[8]}] set_property PACKAGE_PIN U14 [get_ports {out[9]}] set_property PACKAGE_PIN U15 [get_ports {out[10]}] set_property PACKAGE_PIN W18 [get_ports {out[11]}] set_property PACKAGE_PIN V19 [get_ports {out[12]}] set_property PACKAGE_PIN U19 [get_ports {out[13]}] set_property PACKAGE_PIN E19 [get_ports {out[14]}] set_property PACKAGE_PIN U16 [get_ports {out[15]}] set_property IOSTANDARD LVCMOS33 [get_ports clk] set_property PACKAGE_PIN W5 [get_ports clk] - 除法器 （1）除法器代码 module devide #(parameter size=8)( input [size-1:0]a, input [size-1:0]b, output [size-1:0]consult, output [size-1:0]remainder ); reg [2*size-1:0] a_16; reg [2*size-1:0] b_16; always@(*)begin if(b==0)begin a_16={16'b1111_1111_1111_1111}; end else begin a_16={8'b0000_0000,a}; b_16={b,8'b0000_0000}; repeat(size)begin a_16=a_16<<1; if(a_16>=b_16)begin a_16=a_16-b_16+1'b1; end else begin a_16=a_16; end end end end assign consult=a_16[size-1:0]; assign remainder=a_16[size*2-1:size]; endmodule （2）七段译码管控制代码 module led#(parameter size=8)( input clk, input [size-1:0]consult, input [size-1:0]remainder, output[3:0]wei_show, output[7:0]duan_show ); //分频 integer cnt; reg clk_400hz; always@(posedge clk)begin if(cnt==32'd100000)begin cnt<=0; clk_400hz=~clk_400hz; end else begin cnt<=cnt+1'b1; end end reg[3:0]wei_ctrl=4'b1110; always@(posedge clk_400hz)begin wei_ctrl<={wei_ctrl[2:0],wei_ctrl[3]}; end reg [3:0]duan_ctrl; always@(wei_ctrl)begin case(wei_ctrl) 4'b1110: duan_ctrl = consult[3:0]; 4'b1101: duan_ctrl = consult[7:4]; 4'b1011: duan_ctrl = remainder[3:0]; 4'b0111: duan_ctrl = remainder[7:4]; default: duan_ctrl = 4'hf; endcase end reg[7:0]duan; always@(duan_ctrl)begin case(duan_ctrl) 4'h0:duan=8'b1100_0000; //h-a 4'h1:duan=8'b1111_1001; 4'h2:duan=8'b1010_0100; 4'h3:duan=8'b1011_0000; 4'h4:duan=8'b1001_1001; 4'h5:duan=8'b1001_0010; 4'h6:duan=8'b1000_0010; 4'h7:duan=8'b1111_1000; 4'h8:duan=8'b1000_0000; 4'h9:duan=8'b1001_0000; 4'ha:duan=8'b1000_1000; 4'hb:duan=8'b1000_0011; 4'hc:duan=8'b1100_0110; 4'hd:duan=8'b1010_0001; 4'he:duan=8'b1000_0110; 4'hf:duan=8'b1000_1110; default:duan=8'b1100_0000; endcase end assign wei_show = wei_ctrl; assign duan_show = duan; Endmodule （3）顶层模块调用除法器和七段译码管 module main #(parameter size=8)( input clk, input [size-1:0]a, input [size-1:0]b, output [size-1:0]consult, output [size-1:0]remainder, output [3:0]wei_show, output[7:0]duan_show ); devide demo1(.a(a),.b(b),.consult(consult),.remainder(remainder)); led demo2(.clk(clk),.consult(consult),.remainder(remainder),.wei_show(wei_show),.duan_show(duan_show)); endmodule （4）管脚约束文件 set_property IOSTANDARD LVCMOS33 [get_ports {a[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {a[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {a[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {a[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {a[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {a[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {a[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {a[0]}] set_property PACKAGE_PIN R2 [get_ports {a[0]}] set_property PACKAGE_PIN T1 [get_ports {a[1]}] set_property PACKAGE_PIN U1 [get_ports {a[2]}] set_property PACKAGE_PIN W2 [get_ports {a[3]}] set_property PACKAGE_PIN R3 [get_ports {a[4]}] set_property PACKAGE_PIN T2 [get_ports {a[5]}] set_property PACKAGE_PIN T3 [get_ports {a[6]}] set_property PACKAGE_PIN V2 [get_ports {a[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {b[0]}] set_property PACKAGE_PIN W13 [get_ports {b[0]}] set_property PACKAGE_PIN W14 [get_ports {b[1]}] set_property PACKAGE_PIN V15 [get_ports {b[2]}] set_property PACKAGE_PIN W15 [get_ports {b[3]}] set_property PACKAGE_PIN W17 [get_ports {b[4]}] set_property PACKAGE_PIN W16 [get_ports {b[5]}] set_property PACKAGE_PIN V16 [get_ports {b[6]}] set_property PACKAGE_PIN V17 [get_ports {b[7]}] set_property PACKAGE_PIN L1 [get_ports {remainder[0]}] set_property PACKAGE_PIN P1 [get_ports {remainder[1]}] set_property PACKAGE_PIN N3 [get_ports {remainder[2]}] set_property PACKAGE_PIN P3 [get_ports {remainder[3]}] set_property PACKAGE_PIN U3 [get_ports {remainder[4]}] set_property PACKAGE_PIN W3 [get_ports {remainder[5]}] set_property PACKAGE_PIN V3 [get_ports {remainder[6]}] set_property PACKAGE_PIN V13 [get_ports {remainder[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {remainder[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {remainder[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {remainder[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {remainder[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {remainder[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {remainder[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {remainder[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {remainder[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {consult[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {consult[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {consult[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {consult[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {consult[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {consult[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {consult[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {consult[0]}] set_property PACKAGE_PIN V14 [get_ports {consult[0]}] set_property PACKAGE_PIN U14 [get_ports {consult[1]}] set_property PACKAGE_PIN U15 [get_ports {consult[2]}] set_property PACKAGE_PIN W18 [get_ports {consult[3]}] set_property PACKAGE_PIN V19 [get_ports {consult[4]}] set_property PACKAGE_PIN U19 [get_ports {consult[5]}] set_property PACKAGE_PIN E19 [get_ports {consult[6]}] set_property PACKAGE_PIN U16 [get_ports {consult[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {duan_show[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {duan_show[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {duan_show[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {duan_show[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {duan_show[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {duan_show[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {duan_show[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {duan_show[0]}] set_property PACKAGE_PIN W7 [get_ports {duan_show[0]}] set_property PACKAGE_PIN W6 [get_ports {duan_show[1]}] set_property PACKAGE_PIN U8 [get_ports {duan_show[2]}] set_property PACKAGE_PIN V8 [get_ports {duan_show[3]}] set_property PACKAGE_PIN U5 [get_ports {duan_show[4]}] set_property PACKAGE_PIN V5 [get_ports {duan_show[5]}] set_property PACKAGE_PIN U7 [get_ports {duan_show[6]}] set_property PACKAGE_PIN V7 [get_ports {duan_show[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {wei_show[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {wei_show[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {wei_show[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {wei_show[0]}] set_property PACKAGE_PIN U2 [get_ports {wei_show[0]}] set_property PACKAGE_PIN U4 [get_ports {wei_show[1]}] set_property PACKAGE_PIN V4 [get_ports {wei_show[2]}] set_property PACKAGE_PIN W4 [get_ports {wei_show[3]}] set_property IOSTANDARD LVCMOS33 [get_ports clk] set_property PACKAGE_PIN W5 [get_ports clk] |  |  |  |  |  |  |
| 五、实验过程中遇到的问题及解决情况 1.自带的时钟电频太快，无法看清数字。因此在七段译码管中添加了分频器的代码，以便能稳定地看到数字。 2.整数型为4字节，即为32位2进制，但由于板子资源不足，因此我们将其简化为8位二进制。且由于27*27=16384，超过四位七段数码管所能表示的范围，因此将其转化为16进制再显示。 3.除法器最初思路为简单的用两个寄存器表示结果，其中一个表示商s，另一个为余数r，设除数为x,被除数为y,如果x>=y,则用x=x-y,s++，直到x<y,此时余数r=x,商为s。但这个想法会造成资源不必要的浪费，因此经过查阅资料后改进方法。 4.在最初设想中，除0时led应为ffff，ld1-15输出为1，但在实际中却非如此，后来我将h15_15_15_15改为了b1111_1111_1111_1111便成功了。 |  |  |  |  |  |  |
| - 实验结果及分析和（或）源程序调试过程 1.乘法器 （1）仿真文件 - module Try(); - reg [7:0] a; - reg [7:0] b; - wire [15:0] out; - reg clk; - mutl_machine tryone(a,b,out); - always - #50 - clk = ~clk; - initial - begin - a = 8'b00000001; - b = 8'b00000000; - clk = 0; - end - always@(posedge clk) - begin - a = {$random%256}; - b = {$random%256}; - end - endmodule - 仿真结果 - 电路 - 资源利用 - 上板测试结果 - 输入为1*1， 结果为1 - 输入为27*27，结果为4000（16进制）=16384 2.除法器 （1）仿真文件 module sim(); reg clk; reg [7:0] a; reg [7:0] b; wire [7:0] consult; wire [7:0] remainder; main utt(.a(a),.b(b),.consult(consult),.remainder(remainder)); initial begin clk = 0; a = 0; b = 0; end always #0.001 clk=~clk; always@(posedge clk)begin a = {$random%256}; b = {$random%256}; end endmodule （2）仿真结果 （3）电路图 （4）资源利用 （5）上板测试 - b=0 - a=0,b=1 - a=1,b=1 - a=1,b=12 - a=12,b=5 - a=120,b=10 七、小组分工情况说明 孙莹莹：乘法器的设计，上板，实验报告的攥写。 刘淑文：除法器的设计，上板，实验报告的攥写。 |  |  |  |  |  |  |